E

Title	Page No.

INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .1

SAFETY PRECAUTIONS/WARNINGS   . . . . . . . . . . . . . . . . .1

SPECIFICATIONS   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .3

CONTROLS AND INDICATORS  . . . . . . . . . . . . . . . . . . . . . .6

TESTING PROCEDURES   . . . . . . . . . . . . . . . . . . . . . . . . . .7

BATTERY AND FUSE REPLACEMENT   . . . . . . . . . . . . . . .11

MAINTENANCE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .11

SERVICE PROCEDURES   . . . . . . . . . . . . . . . . . . . . . . . . .12

INTRODUCTION

• Congratulations. You have purchased a precision instrument manufactured to the highest quality  standards. This Digital Multimeter is a general-purpose instrument designed for use

in general electronics, home electrical applications, and auto- motive electrical/electronic systems.

• This meter is designed to test or measure AC voltage, DC voltage, batteries, DC current, AC current, resistance, diodes and continuity.

• Please take the time to read these operating instructions thor- oughly and completely. Failure to follow these instructions may result in electrical shock, instrument damage and/or damage

to  the equipment under test. Always use  extreme caution when working on or around electrically operated equipment.

SAFETY PRECAUTIONS/	WARNINGS

Do not operate this multimeter before reading this manual in its entirety. The following guidelines must be followed to avoid acci- dents that can result in electric shock or personal injury.

• Pay close attention to	WARNINGS stamped on the front and rear of the meter's case. These warnings, as well as all warnings and precautions used through out this manual, must

be followed to avoid electric shock and/or personal injury.

• The RESPONSIBLE PARTY shall be made aware that, if the equipment is used in a manner not specified by the manu- facturer, the protection  provided by the equipment may be

impaired.

• Before using any of the functions on this  meter,  verify its proper operation on a known similar function source where the unit value is also known. Take corrective action based on

the indicated results.

To prevent electrical shock and/or damage to the tester or the equipment under test, observe the following safety precautions:

1

E

• DO NOT apply more than the rated voltage, as marked on the meter, between terminals or between any terminal and earth ground.

• Use caution when working above 30V AC rms, 42 V peak, or

60 V DC. Such voltages pose a shock hazard.

• To avoid false readings that could lead to  possible electric shock or personal injury, replace the batteries as soon as the low battery indicator	displays.

• Always  inspect  the  multimeter,  test  leads  and  any  other accessories for damage prior to every use. If any damage is found, do not use tester until repairs are done.

• Always consider electrical and electronic equipment to be ener- gized (live). Never assume any equipment is de-energized.

• Never ground yourself when taking electrical measurements.

Isolate yourself from ground by using dry rubber insulating

mats  to cover all exposed/grounded metal. Stand  on rubber mats and wear dry clothing.

• Never  take  resistance  measurements  on  energized  (live)

electrical or electronic equipment.

• Use one hand, instead of two, whenever  possible to take measurements. If two hands must be used, use extreme cau- tion not to contact any energized conductors with your hands.

Be certain test leads are dry and clean.

• Do not hold the instrument when taking measurements. Place the instrument on a clean, insulating surface prior to taking any measurement.

• Don't become part of the circuit. Think safety. Act safely.

If working on a vehicle, take the following added precautions:

• Only work on vehicle in a well ventilated area.

• Always wear safety eye protection.

• Avoid moving fan blades or any potentially moving parts.

• Avoid hot engine parts.

• Put transmission in "park" (automatic transmission vehicles) or

"neutral" (manual transmission vehicles). Set the parking brake.

• Turn the ignition "off" before connecting or disconnecting any testing equipment.

• Put blocks on drive wheels.

• Avoid wearing loose clothing or jewelry when working on a

vehicle.

• Read your vehicle's service manual and follow it's safety pro- cedures.

2

E

SPECIFICATIONS

GENERAL SPECIFICATIONS AND FEATURES

• 3-½ digit LCD display (maximum reading of 2000); 3 LEDs;

green, yellow and red

• Auto Range (ACV, DCV, Ohm, AC mA, and DC mA)

• Automatic negative (-) polarity indication

• Automatic zero adjustment

• Over range indicator (except 10 A function). Displays "OL" on

LCD

• Low battery indicator. Displays battery symbol	on LCD

• Automatic power shut off (after 15 idle minutes)

• Pollution Degree 2

• Measuring circuit category II

• Operating environment:

Temperature - 32° to 104° F. (0° C to 40° C)

Humidity - Less than 80% relative humidity (non-condensing) Altitude - up to 6562 ft (2000 meters)

• Storage environment:

Temperature - 4° to 140° F (- 20° to 60° C)

Humidity - Less than 90% relative humidity (non-condensing)

• Power Supply: Two 1.5V AA batteries

• Fuse:  315mA/250V  5X20  mm  (Radio  Shack,  GMA/217

series; #270-1046) fuse

• Dimensions:

Height - 5.50 in. (139 mm)

Width - 3.50 in. (89 mm) Depth - 1.25 in. (32 mm)

• Weight (including batteries): approximately 6.3 oz (180 g)

ELECTRICAL SPECIFICATIONS DC VOLTS

| Range | Resolution | Accuracy | Notes |
| --- | --- | --- | --- |
| 200.0mV | 0.1mV | ±(0.8% of reading + 5 digits) | Input Resistance: 10M Overload Protection: 600V DC or AC 600V RMS. |
| 2000V | 1mV |  |  |
| 20.00V | 10mV |  |  |
| 200.0V | 100mV |  |  |
| 600V | 1V |  |  |

3

E

AC VOLTS

| Range | Resolution | Accuracy | Notes |
| --- | --- | --- | --- |
| 2.000V | 1mV | ±(1.2% of reading + 5 digits) | Input Resistance: 10M Overload Protection: 600V DC or AC 600V RMS. Frequency Response: 50Hz - 400Hz |
| 20.00V | 10mV |  |  |
| 200.0V | 100mV |  |  |
| 600V | 1V |  |  |

RESISTANCE (OHMS)

| Range | Resolution | Accuracy | Notes |
| --- | --- | --- | --- |
| 200.0 | 100m | ±(1.0% of reading + 5 digits) | Overload Protection: 250V DC or AC RMS. |
| 2000k | 1 |  |  |
| 20.00k | 10 |  |  |
| 200.0k | 100 |  |  |
| 2.000M | 1k |  |  |
| 20.00M | 10k | ±(2.0% of reading + 5 digits) |  |

DC AMPS

| Range | Resolution | Accuracy | Notes |
| --- | --- | --- | --- |
| 20.00mA | 10 A | ±(1.0% of reading + 5 digits) | Overload Protection: Fuse 315mA/250V Input Voltage Drop: <0.2V. |
| 200.0mA | 100 A |  |  |
| 2.000A | 1mA | ±(2.0% of reading + 5 digits) | Unfused; 15sec maximum Input Voltage Drop: <0.2V. |
| *10.00A | 10mA |  |  |
| *A waiting period of at least 15 minutes is necessary between every 15 second testing period. |  |  |  |

AC AMPS

| Range | Resolution | Accuracy | Notes |
| --- | --- | --- | --- |
| 20.00mA | 10 A | ±(1.5% of reading + 5 digits) | Overload Protection: Fuse 315mA/250V Input Voltage Drop: <0.2V. |
| 200.0mA | 100 A |  |  |

4

E

BATTERY TEST (LEDs)

| When the voltage of the battery being tested is lower than 10% of its rated voltage, the red LED will not light. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Range | Resolution | Load Current | Accuracy | Notes |
| 1.5V | 0.001V | 10mA (approx.) | ±(5% of reading + 5 digits) | Green LED: 1.30V ±0.075V and up. Yellow LED: 0.94V ±0.075V to 1.29V ±0.075V. Red LED:0.15V±0.075V to 0.93V±0.075V |
| 6V | 0.01V | 100mA (approx.) |  | Green LED: 5.22V ±0.3V and up. Yellow LED: 3.76V ±0.3V to 5.21V±0.3V. Red LED:0.6V±0.3V to 3.75V±0.3V |
| 9V | 0.01V | 10mA (approx.) |  | Green LED: 7.83V ±0.45V and up. Yellow LED: 5.64V ±0.45V to 7.82V±0.45V. Red LED:0.9V±0.45V to 5.63V±0.45V |
| 12V | 0.01V | 200mA (approx.) |  | Green LED: 10.44V ±0.6V and up. Yellow LED: 7.52V ±0.6V to 10.43V±0.6V. Red LED:1.2V±0.6V to 7.51V±0.6V |

DIODE/CONTINUITY TESTS

| Function | Range | Resolution | Description | Note |
| --- | --- | --- | --- | --- |
| Diode Test | 2V | 1mV | Test Current: 1±0.6mA Test Voltage: Approx. 1.5V | Overload Protection: 250V DC or AC RMS |
| Continuity Test | 200 | 0.1 | Approx. 120 or less, buzzer (beeper) will sound |  |

5

E

CONTROLS AND INDICATORS

1.    DCV Function: Measures DC

volts. Auto-ranging from 0 to

600 volts (10 M    impedance)

2.    ACV Function: Measures AC	13

volts. Auto-ranging  from  0 to

1

15

600 volts (10 M   impedance)	2

3.    Resistance	Function:	12	3

14

Measures  resistance.  Auto-		4 ranging  from  0  to  20M	11	5 (20,000,000   ).	10	6

4.    ACmA Function.  Measures AC current in milliamps. One range: 0 to 200 milliamps.

9	8	7

5.    DIODE Function	:  For

testing diodes.

6.    CONTINUITY Function	:  Tests for continuity  between two points.

7.    Volts, mA, OHMS, BAT, DIODE and CONTINUITY Input

Jack.

8.    COM Input Jack:  Common Input Jack.

9.    DC10A Input Jack. For red test lead probe  connection when measuring high DC current (up to 10 amps only).

10. DC10A  Function:  Measures  DC  current.  One  range:

10Amps (DC from 0 to 10 amperes). Unfused.

11. DCmA Function: Measures DC current in Milliamps. One range: 0 to 200 milliamps.

12. Battery Test Function: Four ranges and three LEDS (green, yellow and red). Tests small batteries; 1.5V, 6V, 9V and 12V.

13. OFF Function:  Turns unit "off" when function is selected.

14. Function/Range Selector Switch: Selects desired function or range.

15. Liquid Crystal Display (LCD):  Displays results of tests or measurements.

Preparation and Caution Before Use

• Inspect the digital multimeter for damage to the case. Do not use if cracked, distorted, excessively dirty or any abnormal condition exists.

• Inspect the test leads for damage. Check for cracked insula- tion, broken or damaged probes, loose or bent probe pins. Do not use if any abnormal condition exists.

• Set the Function/Range Selector Switch to the proper range

BEFORE  taking measurements. If the range/function must

6

E

be  switched during a test, ALWAYS remove  the  test leads from the circuit being measured before switching settings.

• To avoid possible electric shock, instrument damage, and/or equipment damage when taking voltage or current measure- ments, DO NOT exceed the maximum value of the selected

range.

• If the unit is used near high noise Radio Frequency (RF) gen- erating equipment (spark plug wires, ignition coils or alterna- tor), the display may become unstable or indicate large errors.

If you obtain erratic readings during use, isolate the Multimeter as far away as possible from these components.

TESTING PROCEDURES

A.  AC/DC VOLTAGE MEASUREMENT

WARNING

To  avoid  possible  electric  shock,  instrument  damage and/or equipment damage, DO NOT attempt to measure voltages ABOVE 600V AC/DC or take  measurements if the voltage is unknown. 600V AC/DC between the COM and V jacks is the maximum voltage that this instrument is designed to measure. The "COM"  terminal potential should not exceed 300V AC/DC measured to ground.

1.    Plug the BLACK test lead into "COM" jack of the multimeter;

plug the RED test lead into the "V" jack.

2.    Set  the  meter’s  Function/Range  Selector  Switch  to  the appropriate ACV or DCV position as desired (see Controls and Indicators, Items 1 and 2).

3.    Place the RED test lead onto the positive (+) side of the item being tested and the BLACK test lead onto the negative (-) (across the source/load) side of the item. BE CAREFUL not to touch any energized conductors with any part of your body.

4.    Read the results on the display.

B.  RESISTANCE MEASUREMENT	(OHMS)

WARNING

Resistance  measurements must be made on  "de-ener- gized" (dead) circuits ONLY. Impressing a voltage across the multimeter's  terminals while set to any resistance range  may result in electric shock, instrument damage and/or damage to  equipment under test. MAKE SURE equipment is completely de-energized before taking any resistance measurements.

7

E

1.    Plug the RED test lead into the "  " jack of the multimeter;

plug the BLACK test lead into the "COM" jack.

2.    Set the meter’s Function/Range Selector Switch to the "  " range function (see Controls and Indicators, Item 3).

NOTE: To obtain accurate readings, disconnect  at least one side of the item under test from the circuit or circuit board before measuring resistance.

3.    Place the RED test lead onto one side of the item being test- ed and the BLACK test lead onto the other side of the item. (Polarity does not matter when checking resistance).

4.    Read the results on the display.

C. DIODE TEST

WARNING

To avoid electrical shock and/or damage to the multimeter, ensure the power is removed from the circuit before any DIODE testing procedure is conducted. Test diodes on de- energized (dead) circuits only, never on live circuits.

NOTE: A diode is a semiconductor device that lets current flow in one direction only. If the diode to be tested is part of a circuit (with other electronic components), you must isolate it from the other components by disconnecting at least one side of it from the circuit before testing. A good diode will show a low voltage drop across its junction (0.5-0.8 volts for a silicon diode or about

0.3V for a germanium diode) when the leads are connected in one polarity and a very high resistance (or open circuit) when the leads are reversed (connected in the opposite polarity).

1.    Plug the RED test lead into the	jack of the multimeter;

plug the BLACK test lead into the "COM" jack.

2.    Set the meter’s Function/Range Selector Switch to the position (see Controls and Indicators, Item 5).

3.    Place the RED test lead onto one side of the diode being tested and the BLACK test lead onto the other side.

4.    Read the results on the display.

5.    Reverse the test leads and again read the results on the dis- play. Compare the two readings. One reading should indi- cate a voltage drop value; the other reading should indicate an overrange (OL) condition. See note above.

8

E

D. CONTINUITY TEST

WARNING

To avoid electric shock, shut off the power to the test arti- cle before testing it for continuity.

1.    Plug the RED test lead into the	jack of the multimeter;

plug the BLACK test lead into the "COM" jack.

2.    Set the meter’s Function/Range Selector Switch to the position (see Controls and Indicators, Item 6).

3.    Place the RED Test Lead to one end of the wire or device being tested for continuity and the BLACK Test Lead to the opposite end.

4.    Listen to the sound of the beeper and confirm the results by reading the display.

NOTE: The beeper will sound only if the continuity of the item under test (resistance  between the two test leads) measures less than 120 ohms.

E.  AC/DC CURRENT MEASUREMENT (AMPS)

WARNING

To  prevent  electrical  shock  when  performing  current measurements, follow  all  steps as indicated below DO NOT skip any steps or take any short cuts.

The DC10A range is not fused. To avoid current hazard and/or damage to the tester, DO NOT try to take meas- urements on circuits that have more than 10 amps. DO NOT take more than 10 seconds to take the reading. A waiting period of AT LEAST 15  MINUTES is necessary between every 15 second testing period.

1.    Plug the RED test lead into the "mA" or the "DC10A" jack of the multimeter, as applicable; plug the BLACK test lead to the "COM" jack.

2.    Set  the  meter's  Function/Range  Selector  Switch  to  the appropriate Amps range position as desired (see Controls and Indicators, Items 4, 10 and 11).

• To measure from 0 to 200mA, set the Selector Switch to the "DCmA" or "ACmA" position, as applicable.

• To  measure  from  200mA  to  10  Amps  DC,  set  the

Selector Switch to the "DC10A" position.

3.    Disconnect the battery, or shut off the power to the circuit being tested.

9

E

NOTE: To measure current on a particular circuit, you must open up the circuit and connect the test leads in series with the circuit before a reading can be obtained.

4.    Disconnect one end of the wire or device, from the circuit where current will be measured.

5.    Place the RED test lead on the disconnected wire and place the BLACK test lead at the location from which the wire was disconnected (series connection).

6.    Reconnect the battery, or apply power to the circuit being tested.

7.    Read the results on the display.

CAUTION: After the test is completed, shut the power off to the circuit before removing the test leads and before recon- necting any disconnected wires or devices.

NOTE: If the reading obtained is a negative number, reverse the test leads.

F. BATTERY TEST

1.    Plug the RED test lead into the "BAT." jack of the multime- ter; plug the  BLACK test lead into the "COM" jack.

2.    Determine the voltage rating of the battery to be tested.

3.    Set  the  meter’s  Function/Range  Selector  Switch  to  the desired "Battery Test" range (see Controls and Indicators, Item 12).

4.    Place the RED test lead on the positive post of the battery under test; place the BLACK lead on the negative post.

NOTE:  If a battery is completely dead, or nearly dead (below

10% of its rated voltage), the red LED will not light.

5.    Read the results using the display and the LEDs:

• Green	= fully charged (good battery)

• Yellow	= low (questionable)

• Red	= discharged (bad battery)

NOTE:  Battery  life  is  directly  proportional  to   the  current

draw/load of the device that the battery is powering. The three LEDs on the multimeter represent battery state-of-charge aver- ages for the most commonly used devices.

10

E

BATTERY AND FUSE REPLACEMENT

WARNING

When replacing the battery or the fuse, remove only the rear  panel. Do  not  remove  or  disassemble  the  circuit board or the front panel, these items are not serviceable and if disassembled there is the possibility of loose metal parts shorting the circuit board and causing an electrocu- tion danger to the user.

1.    Turn the Digital Multimeter “OFF” and remove the test leads.

2.    Remove the two screws on the back of the meter and sep- arate the case.

3.    Replace the fuse or batteries as necessary:

• For  battery replacement: Remove the  batteries from the battery compartment and replace only with two AA (1-½ volt) alkaline batteries.

• For fuse replacement: Remove the fuse from the fuse holder and replace with a  0.315A/250V - UL Listed

Bussmann,  GMA  Type  (Radio  Shack   GMA/270 series; #270-1046 ) fuse.

NOTE:  Use  a  0.315A/250V,  5x20mm  type   fuse  ONLY  - Bussmann, GMA Type (Radio  Shack #270-1046 or similar). Using an incorrect fuse may result in serious injury and/or damage to the unit.

4.    Reassemble the case and secure with the two screws.

MAINTENANCE

1.    No periodic maintenance is required other than the replace- ment of the battery, the fuse, and visual inspection of the meter.

2.    Keep the meter clean and dry. DO NOT use solvent to clean, use a damp (not wet) cloth and fully dry after cleaning.

3.    The  only  replaceable  parts  are  the  1.5   AA   batteries,

0.315A/250V   fuse  (see  page  10  for  battery   and  fuse replacement) and the Test Leads (for Test Leads call serv- ice department).

11

E

SERVICE PROCEDURES

The Manufacturer warrants to the original purchaser that this unit is free of defects in materials and workmanship under normal use and maintenance for a period of one (1) year from the date of orig- inal purchase. If the unit fails within the one (1) year period, it will be  repaired  or  replaced,  at  the  Manufacturer's  option,  at  no charge, when returned prepaid to the Technical Service Center with Proof of Purchase. The sales receipt may be used for this pur- pose. Installation labor is not covered under this warranty.

All  replacement  parts,  whether  new  or   re-manufactured, assume as their warranty period for only the remaining time of this warranty. This warranty does not apply to damage caused by  improper use, accident, abuse, improper  voltage,  service, fire, flood, lightning, or other acts of God, or if the product was altered or repaired  by anyone other than the Manufacturer's Technical Service Center. Consequential and  incidental dam- ages are not recoverable under this warranty. Some states do not allow the exclusion or limitation of incidental or consequen- tial damages, so the above limitation or exclusion may not apply to you.

This warranty gives you specific legal rights, and you may also have other rights, which vary from state to state. No portion of this warranty may be copied or duplicated without the expressed written permission from the Manufacturer.

Obtaining Warranty Service:

Products requiring service should be returned as follows:

1.    Call  the  Technical  Service  Center  to   obtain   a  Return

Reference Number:

USA & Canada = 1-800-544-4124

Other = 714-241-6805

2.    Package the product carefully to prevent shipping damage

3.    Include  your  name,  return  address,  and  a  day  contact phone

4.    Enclose a copy of the dated sales receipt

5.    Describe the problem

6.  Ship prepaid to: Technical Service Center,   17291  Mt.

Herrmann Street, Fountain Valley, CA 92708  U.S.A.

Phone: 1-800-544-4124 or 714-241-6805 Fax: 714-432-7910

Fax: 714-432-7910

Web:  www.iEQUUS.com

Email:  service@iEQUUS.com

12

E

E

E

E